WHATVGA version 2.00  29/Sep/95

Copyright 1991-95   Finn Thgersen

All Rights Reserved

Command line switches:

whatvga [+acumos] [-ncr] [=1024] [/a]

where +chipset  forces WHATVGA to proceed as if this chipset was found.

The test for this chip set will be executed in order

to check for memory and revision.

No other chipsets will be tested.

-chipset  ensures that the tests for this chipset will NOT be

executed. Useful if a chipset is misidentified

or some earlier test gives problems.

=kbytes   Forces the number of Kbytes of video RAM on the

adapter to the specified value.

/a        Run automatic test and write results to a file.

The user responds with Y or N to each test and can

enter comments to the individual tests.

On startup you may be asked to select the video system

to test (say if you have a VESA driver loaded).

If the program seems to halt for more than a few seconds

try to press the enter key.

The recommanded way to use the program is to start it

with no command line parameters and play with it a bit.

See that the chipset, memory and DAC are ID'd properly

and that you can display at least some super VGA modes.

Then run the program with /a to do the full test.

For the /a test, the system should be as "clean" as

possible, as TSRs (including mouse and keyboard drivers)

and memory managers can sometimes affect the results.

Try booting from a floppy.

Comments should only be entered when something is wrong

or needs clarification. Ie. there is no need to enter

"Fine" or "All ok" thats what the yes/no queries are for.

When something is wrong try to be as descriptive as

possible (remember, I can't see the screen).

/c        Disabled clock detection. Clock detection can lock up

some systems. For instance S3 systems will lock up if

a 0Hz clock is selected.

/d        Debug option. display the name of each chipset before it

is tested.

/i        Ignore the configuration file (WHATVGA.CFG)

/p        PCI test. Disables the test for PCI devices. Note that

WHATVGA has not been tested with with PCI type 2 busses.

Note that if you use the + - or = commandline options the program should

pause, displaying:

Forced to .....

Press a key to continue

If this does NOT happen the option(s) probably were not recognised.

The options can also be set from a configuration file. On startup WHATVGA

checks for the file WHATVGA.CFG in the current directory. The file consists of

the general options at the top and then the chipset specific options in one

section for each chipset started by a [chipset] header.

Most options are of the keyword=value type. The keyword is not case-sensitive.

For Yes/No options a value of YES, Y, ON or 1 (all not case-sensitive) will

be takes as YES, all other values as NO. Leading and trailing spaces are

ignored. The command line switches will win out if there is a conflict.

General Options above the first chipset header:

AutoTest =Yes/No      ;Enables the Auto Test feature if set to YES

ClockTest=Yes/No      ;Disables Clock testing if set to NO

Debug    =Yes/No      ;Enables debug output if set to YES

PCItest  =Yes/No      ;Disables the PCI test if set to NO (same as /p).

Memory   =number      ;Forces the amount of video memory (in Kbytes)

Ignore   =chipset     ;Causes WHATVGA to skip the tests for this chipset.

Any number of IGNORE= lines can be added.

Chipset  =chipset     ;Forces WHATVGA to assume the chipset was found

Sample file:

Debug=Off                   ;No DEBUG option

AutoTest=0                  ;No AutoTest

ClockTest=N                 ;Disable clock test

Memory=1024                 ;Force video Memory to 1Mb

[S3]                        ;S3 chipset section

-103                        ;Disable mode 103h

+4322 732 498 P8 1000       ;Adds mode 4322h, a 732x498 P8 mode with 1000

; bytes per scanline (use 0 bytes/scanline for

; default width)

If you have problems with the identification/startup:

- The screen goes blank or all white...

Try to run: WHATVGA /d

This will display the name of each chipset and waits for

you to press the Enter key BEFORE the chipset is tested.

When you have found the offending chipsets, try to run:

WHATVGA -Chips&Tech     (assuming Chips&Tech is the culprit)

Report the problem.

- The chipset is misidentified.

Try to run: WHATVGA -chipset

where chipset is one of the following:

Chips&Tech    Chips & Technologies

Paradise      Paradise/Western Digital

Video7        Video7

Tseng         Tseng Et3000, ET4000, ET4000/W32

Trident       Trident 8800/8900/9000...

Everex        Everex

ATI           ATI VGA Wonder/Graphics Ultra +/Pro

Genoa         Genoa 6x00 (not 5x00, 7x00 or 8x00)

Oak           Oak 37/67/77/87

Cirrus        Cirrus 5x0/6x0

Ahead         Ahead A&B

NCR           NCR 77c2x

Yamaha        Yamaha 6388

S3            S3 86c series

ALG           Avance Logic ALG series

MXIC          MXIC 860x0

Realtek       Realtek

PRIMUS        Primus P2000

Cirrus54      Cirrus CL-GD54xx

Cirrus64      Cirrus CL-GD64xx

Weitek        Weitek W5x86

COMPAQ        Compaq IVGA/AVGA/QVision

UMC           UMC 85c408, 85c418

HMC           HMC HM86304

WeitekP9      Weitek P9x00 series

XGA           XGA compatibles

AGX           IIT AGX series

Mach32        ATI Mach32

Mach64        ATI Mach64

Matrox        Matrox VGA

ARK           ARK Logic

Acer          Acer M31x5

Sierra        Sierra SC15064 (Falcon/64)

MediaVision   Media Vision Pro Graphics

MGA           Matrox MGA Accelerator

Alliance      Alliance ProMotion

- Memory Size is identified wrong.

Run WHATVGA =2048    (or how many Kbytes you have)

Report the problem.

Whatvga has several options in the main menu:

1 Test standard VGA

Allows you to test the standard VGA video modes.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

This option is not shown for non-VGA devices.

See 2) for the details.

2 Test extended modes

Allows you to test the extended video modes of the adapter.

Draws a test pattern, including a color palette

and writes basic information as text on the display.

On the upper and left edge every hundred pixels is marked.

when you press a key the testpattern is drawn again, this time

in the largest size the available video memory will support

("Virtual Screen"), allowing you to scroll around in the display

using the arrow keys. You should be able to move in the entire

pattern, but not to move so far right or down that the display wraps.

If you can move past the right edge (or can't reach it) then the

scroll function probably needs a factor 2 (divide or multiply)

in the SetVStart function.

When you press the Enter key a textscreen is shown with the

statistics for the mode, this screen also shown if the rutine

to analyse the mode from the register values (AnalyseMode) reaches

the same result. If not the values are shown as "Calculated".

By pressing "d" or "D" you can see a dump of all the VGA (and extended)

registers. The registers can be saved (appended) to the file REGISTER.VGA

by pressing "f" or "F".

3 Search for modes

Sequentially attempts to set modes 14h to 7Fh, each time testing

if the mode was succesfully set. If so the testpattern is drawn

and the mode is analysed.

5 Hardware Cursor test

This item attempts to place a 32x32 hardware Cursor in the

selected mode. The cursor can be moved around on the screen

with the arrow keys.

6 BitBLT test

This item first draws the testpattern, then draws the color palette with

BitBLT operations at the centre of the display, then makes four blockcopies

via BitBLT of the central palette, 90 degrees apart, each slightly

overlapping the original. As the edges of the source is modified during

the copying these changes will (in part) be copied to the new images.

7 Line Draw test

This item attempts to draw lines from the center of the screen

in a "wheel" using different colors.

8 Read/Write bank test

This item copies the left half of the screen to the right half, reversing

the line order, thus copying from "low" to "high" banks using seperate

read/write banks.

0 Exit

Items 5-8 are only present if WHATVGA believes the adapter supports the

function, though you can always try the test by pressing the key.

Special functions for test purposes (may hang your PC or worse):

B Register Bit test.

You are asked for the base register (such as 3C0h, 3C4h, 3CEh or

3d4h), starting index and starting bit number (0-7).

Then you select the mode for the test.

Now the test pattern is drawn in the desired mode, but using the

entire video memory (like the "virtual screen").

The selected bit will be changed for ~0.5 sec, then restored for

another 0.5 sec. This repeats until another bit is selected by

-, + or * or the test is stopped by the Esc key.

- steps one bit back, + one forward - both changing the index if

necessary. * advances to bit 0 of the next index.

This can be used to figure out the function of individual bits, the

Display Start extension bits being the obvious example (and the

intended target), but also clock bits etc.

!!NOTE!! this may hang your PC or expose your video card or monitor

to conditions beoynd their intended operational specifications.

While damage to your monitor or video card is extremely unlikely

(I haven't heard of a confirmed case since the old IBM MDA monitor),

it is theoretically possible and my insurence company refuses to

recognize any responsibility what so ever :-).

Hanging your PC is however a very real possibility.

Note the bit you were testing, reboot (power off may be required)

and restart from the NEXT bit.

D DAC test submenu

Tests various special DAC functions:

2 - Test 24bit (16Mcolor) mode

3 - Test 32bit (16Mcolor RGBa) mode

5 - Test 15bit (32Kcolor) mode

6 - Test 16bit (64Kcolor) mode

Tests the switching between 256color mode and 15/16/24/32bit mode.

Besides the normal 256color test pattern a RGB color scale in the

selected mode is also shown (this will look weird in 256color mode)

Pressing + toggles between the two modes.

Esc or Enter terminates the test.

8 - Test 6/8bit mode

A test pattern is drawn in the selected 256color mode. The palette

is set up to use the full 256 of 16m palette (256 shades of each

basic color). When the DAC is in 6bit mode there are only 64 shades,

thus the color bars will "break". The + key switches between 6bit

and 8bit DAC mode. Esc or Enter terminates the test.

A - DAC Cmd register Analysis

This test examines the DAC command register which is accessed by

reading 3C6h several times. Note that only some DACs have this

register! The PEL register (3C6h) is set to 4 different patterns

(55h, AAh, 5Ah and A5h) and for each pattern 3C6h is read 8 times.

If the DAC has a command register it will show after 4 or 5 reads.

Some DACs have more than one "hidden" register at 3C6h and some DACs

revert to the PEL register after the first read of the command

register, while others allow multiple reads of the command register.

Then the command register is tested for read/writability, first bit

by bit (0 to 7) this is reported as: "Dac Single Bit Mask: EF" where

EF indicates that all but bit 4 is Read/Writable, then all values (0

to 255) are tested and reported as: "DAC mask: AA R/W: BB Change: CC

Set: DD Clear: EE" where AA shows the bits actually tested - FFh =

all (Some DACs have problems with some bits combinations), BB are the

Read/Writable bits, CC the ones that changed but aren't R/W, DD the

bits that are always set and EE the ones that are always clear.

If any patterns causes unexpected results they are shown as: "  AA =

BB" where AA is the value written and BB the one read back.

All fields above are in hex.

C - Test Command register

Allows you to test the behaviour of the DAC command register. Apart

from the usual 256color test pattern a number of color scales are

drawn in (from top to bottom) 15, 16, 24 and 32 bit color format.

These will of course look hopeless in 256color mode, but allows you

to detect if you switch to one of these modes. The F1 - F8 keys will

toggle bits 0 - 7 of the command register. The value you are

currently trying to program, is shown at the top.

G - Gamma Test

Allows you to test if your DAC can handle gamma correction in Hi-

and True color modes. You are asked to select a Hi- or Truecolor

mode, the test pattern is drawn and now one of the basic colors

(Red, Green and Blue) will be inverted every time + is pressed.

M - Select base mode

Selects the mode to use for the DAC tests from the available 256

color modes. By default the 640x480 mode is used.

A test pattern is drawn in the selected 256color mode.

The palette is set up to use the full 256 of 16m palette (256

shades of each basic color). When the DAC is in 6bit mode there

are only 64 shades, thus the color bars will "break".

The keys 6 and 8 switches between 6bit and 8bit DAC mode

(if the 8bit DAC mode works :-) ).

Esc or Enter terminater the test.

Currently this only works for the AT&T (20c490/491), Sierra 24bit

(SC15021/25), Trident TDK8001 and Brooktree (Bt477,Bt484/5) DACs.

R Register Read/Write test.

You are asked for the base register (such as 3C0h, 3C4h, 3CEh or 3d4h),

the mask used for the index register. The mask is entered as an 8 character

string where each character controls one bit of the index (bit 0 first) and

can be 'x' (don't care), '0' (always 0), '1' (always 1) or ' ' (don't care

if the bit is Read/Writable in the index register, 0 or 1 if it is always 0

or 1). Any other characters are interpreted as ' '.

Finally you are asked for the mode to use for the test. After the test a

table showing the status of each bit tested is output starting from index 0

bit 0. Each bit is shown as: '.' (Masked), '0' (Always 0), '1' (Always 1),

'A' (Automatically changing), 'C' (Read/Writable, but changed by some other

bit) or 'W' (Read/Writable).

The tests:

ID:      Is the Chip Set, DAC and video memory identified correctly?

Xres:    Are the modes (standard and extended) displayed correctly?

Display type, Switch settings and BIOS versions can cause

some modes to be unavailable or out of sync.

Scrl:    Can you display and scroll in the Virtual Screen ?

You should be able to move in the entire virtual screen,

but not to move so far right or down that the display wraps.

If you can move past the right edge (or can't reach it)

we probably miss a factor of two in the SetVStart function.

Also can you scroll horizontally in 1 pixel steps ?

If the display wraps when the start crosses the middle (or 1/4)

of memory we have a problem with the high bits (the Y variable)

in the SetVStart function or with an enable bit.

Also part of video memory can be inaccessible in modes that

normally only use <half the memory causing the testpattern to

be overwritten.

Anal:    Are the modes (particularly the extended ones) analysed

correctly (Ie do the "Calc:" fields match the "List:" ones) ?

Some modes, such as 2 and 4 color and mode 13h are hard to ID

on many adapters.

Cur:     Is the HardWare cursor shown correctly and can it be moved

around the display with the arrow keys ?

The cursor should look like a snipers sights with a dot in the

upper left corner.

Blit:    Is the central palette drawn correctly ? Are the secondary palettes

copied correctly ?

Line:    Are the lines drawn from the middle to fill a rectangle ?

RWbank:  Do the two halfs of the screen match ?

Adapters tested:

Mem: ID: Xres: Scrl:  Anal:  Cur:  Blit:  Line:  RWbank:

Acer:

M3135

M3145

Acumos:  See Cirrus 5401 and 5402

AVGA2 (5402)  512 Ok  Ok    Ok     Ok     Ok*   NA     NA     NA

Ahead:          .   .   .     .      .      .     .      .      .

V5000 A

V5000 B       1M  Ok  Ok    Ok     Ok     NA    NA     NA     Ok

Alliance (ProMotion):

3210          1M  Ok  Ok    Ok     Ok

Appian:         .   .   .     .      .      .     .      .      .

AGC-98032     2M  No

ARK Logic:      .   .   .     .      .      .     .      .      .

ARK1000VL     2M  Ok  Ok    Ok*    Ok     Ok    No     No     Ok

ARK1000PV

ARK2000PV     2M  Ok  Ok    Ok*    Ok     Ok    No     No     Ok

ATI:            .   .   .     .      .      .     .      .      .

18800                                     NA    NA     NA

18800-1       512 Ok  Ok    Ok     Ok     NA    NA     NA     Ok

28800-2                                   NA    NA     NA

28800-4                                   NA    NA     NA

28800-5                                   NA    NA     NA

28800-6       1M  Ok  Ok    Ok     Ok     NA    NA     NA     Ok

68800-3 (VGA):2M  Ok  Ok*   Ok     Ok     NA    NA     NA     Ok

(ACC):2M  Ok  Ok    Ok     Ok*    Ok    Ok*    Ok*    Ok

68800-6

68800-LX

68800-AX

88000GX (VGA):1M* Ok  Ok    Ok*    Ok     NA    NA     NA     Ok

(ACC):4M  Ok  Ok*   Ok*    Ok     Ok    Ok*    Ok*    Ok

Avance Logic:   .   .   .     .      .      .     .      .      .

ALG2101       1M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    Ok

ALG2201       1M  Ok  Ok    Ok     Ok     No    No     No     Ok

ALG2228           Ok  Ok    Ok     Ok     No    No     No     Ok

ALG2301       2M  Ok  Ok    Ok     Ok     No    No     No     Ok

Chips&Tech:     .   .   .     .      .      .     .      .      .

82c450        512 Ok  Ok    Ok     Ok     NA    NA     NA     NA

82c451                                    NA

82c452        512 Ok  Ok    Ok     OK     Ok    NA     NA     NA

82c453

82c455

82c456

82c457

F65510

F65520

F65530

F65535

F65540

F65545

F64200

F64300        2M  Ok  Ok    Ok     Ok     Ok                  NA

Cirrus:         .   .   .     .      .      .     .      .      .

510/520                                         NA     NA     NA

610/620                                         NA     NA     NA

5401(AVGA1)   256 No                            NA     NA     NA

5402(AVGA2)   512 Ok* Ok    Ok     Ok     Ok*   NA     NA     NA

5410                                            NA     NA     NA

5420          512 Ok  Ok    Ok     Ok     Ok*   NA     NA     NA

5422                                            NA     NA     NA

5424                                            NA     NA     NA

5426          2M  Ok  Ok    Ok     Ok     Ok*   Ok*    NA     NA

5428          1M  Ok  Ok    Ok     Ok     Ok*   Ok*    NA     NA

5429                                                   NA     NA

5430          2M  Ok  Ok*   Ok     Ok     Ok*   Ok*    NA     NA

5434          1M  Ok  Ok    Ok     Ok     Ok*   Ok*    NA     NA

5452

6205                                      NA    NA     NA     NA

6215

6225

6235

6410          512 Ok  Ok    Ok     Ok*    NA    NA     NA     NA

6412

6420

6440

Compaq:         .   .   .     .      .      .     .      .      .

IVGS

AVGA          512 Ok  Ok    Ok     Ok     NA    Ok*    NA     NA

QVGA

QVGA1024

QVGA1280      2M  Ok  Ok*   Ok*    Ok*    Ok*   Ok*           NA

Everex:

Genoa:          .   .   .     .      .      .     .      .      .

51/5200

53/5400

6100

62/6300

64/6600

7900

HMC:            .   .   .     .      .      .     .      .      .

HM86304       512 Ok  Ok    Ok*    Ok     NA    NA     NA     NA

HM86314

IIT:            .   .   .     .      .      .     .      .      .

AGX-xx (VGA): 1M* Ok  Ok    Ok     Ok     NA    NA     NA     NA

AGX10/11

AGX-14        1M  Ok  Ok    Ok*    Ok     Ok    Ok     Ok     NA

AGX-15        2M  Ok  Ok    Ok*    Ok     Ok    Ok     Ok     NA

AGX-16        2M  Ok  Ok    Ok*    Ok     Ok    Ok     Ok     NA

Matrox:         .   .   .     .      .      .     .      .      .

Impres (VGA):

(ACC):

Ultima (VGA):     Ok  Ok*   Ok     Ok

(ACC): 2M  Ok*

Imp Pl (VGA):     Ok  Ok*   Ok     Ok

(ACC): 2M

Media Vision:   .   .   .     .      .      .     .      .      .

PG 1024       2M Ok*

PG 1280       4M  No

MXIC:           .   .   .     .      .      .     .      .      .

86000         512 Ok  Ok    Ok     Ok     NA    NA     NA     NA

86010

86100

NCR:            .   .   .     .      .      .     .      .      .

77C20

77C21

77C22E        4M  Ok  Ok    Ok     OK     Ok    NA     NA     Ok

77C22E+

77C32BLT      2M  Ok  Ok    Ok     Ok     Ok    Ok     NA     Ok

Oak:            .   .   .     .      .      .     .      .      .

OTI-37C       256 Ok  Ok!   Ok     OK     NA    NA     NA

OTI-067       256 Ok  Ok!   Ok     OK     NA    NA     NA

OTI-077       1M  Ok  Ok!   Ok     Ok     NA    NA     NA     Ok

OTI-087       1M  Ok  Ok!   Ok     Ok     Ok*                 Ok

Paradise/WD:    .   .   .     .      .      .     .      .      .

PVGA1A        512 Ok  Ok    Ok     Ok     NA    NA     NA     NA

WD90C00                                   NA    NA     NA

WD90C10                                   NA    NA     NA

WD90C11       512 Ok  Ok    Ok     Ok     NA    NA     NA     Ok

WD90c2x

WD90C24       1M  Ok  Ok    Ok     Ok     Ok*   Ok            Ok

WD90c26

WD90C30       1M  Ok  Ok    Ok     Ok     NA    NA     NA     Ok

WD90C31       1M  Ok  Ok    OK     Ok     Ok*   Ok*    NA     Ok

WD90c33       2M  Ok  Ok    Ok     Ok     Ok    Ok*    Ok*    Ok

Primus:         .   .   .     .      .      .     .      .      .

P2000         2M  Ok  Ok*   Ok     Ok     NA    Ok*    NA     Ok

Realtek:        .   .   .     .      .      .     .      .      .

RT3105        512 Ok  Ok    Ok     Ok     NA    NA     NA     Ok

RT3106                                    NA    NA     NA

S3:             .   .   .     .      .      .     .      .      .

86c911                                                        NA

86c911A/24    1M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c801        2M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c805                                                        NA

86c805p       1M  Ok  Ok    Ok     Ok*    Ok*   Ok*    Ok*    NA

86c928        4M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c864                                                        NA

86c864P       2M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c964                                                        NA

86c964P       2M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c732

86c764

86c764P       1M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    NA

86c866

86c868

86c968

Sierra:

SC15064

SiS:

SG86c201      1M  Ok  Ok    Ok     Ok     Ok                  Ok

Trident:        .   .   .     .      .      .     .      .      .

8800BR                                    NA    NA     NA     NA

8800CS        512 Ok  Ok    Ok     Ok     NA    NA     NA     NA

8900B         1M  Ok  Ok    Ok     Ok     NA    NA     NA     NA

8900C         1M  Ok  Ok    Ok     Ok     NA    NA     NA     NA

9000          512 Ok  Ok    Ok     Ok     NA    NA     NA     NA

9000B                                     NA    NA     NA     NA

9000i         512 Ok  Ok*   Ok     Ok     NA    NA     NA     NA

9000C         512 Ok  Ok    Ok     Ok*    NA    NA     NA     Ok

8900CL                                    NA    NA     NA

8900D         1M  Ok  Ok    Ok     Ok*    NA    NA     NA     Ok

9200CXr       1M  Ok  Ok    Ok     Ok*    NA    NA     NA     Ok

9400CXi       2M  Ok  Ok    Ok     Ok*    NA    NA     NA     Ok

9420DGi       1M  Ok  Ok    Ok     Ok*    NA                  Ok

9440AGi       1M  Ok  Ok    Ok     Ok     Ok                  Ok

9660XGi

Tseng:          .   .   .     .      .      .     .      .      .

ET3000        512 Ok  Ok*   Ok     Ok     NA    NA     NA     Ok

ET4000        1M  Ok  Ok    Ok     Ok     NA    NA     NA     Ok

ET4000W32     1M  Ok  Ok    Ok     OK     Ok*   Ok*    NA     Ok

ET4000W32i A                                           NA

ET4000W32i B  2M  Ok  Ok    Ok     Ok     Ok*   No     NA     Ok

ET4000W32p A  2M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    Ok

ET4000W32p B  2M  Ok  Ok    Ok     Ok*    Ok*   Ok*    Ok*    Ok

ET4000W32p C

ET4000W32p D  1M  Ok  Ok    Ok     Ok     Ok*   Ok*    Ok*    Ok

UMC:            .   .   .     .      .      .     .      .      .

UMC85c408     512 Ok  Ok    Ok     Ok     NA    NA     NA     Ok

UMC85c418     1M  Ok  Ok    Ok     Ok     NA    NA     NA     Ok

Video7/Headland:    .   .     .      .      .     .      .      .

G2-GC205      256 Ok                                          NA

HT-208        512 Ok  Ok    Ok     Ok     Ok*   NA     NA     NA

HT-209

HT-216        1M  Ok  Ok    Ok     Ok     Ok!   NA     NA     NA

Weitek:         .   .     .      .      .     .      .      .

W5086

W5186         256 Ok  Ok*   No     Ok

W5286         1M  Ok  Ok*   No     Ok                         Ok

P9000         2M  No

P9100 (VGA)   2M  Ok  Ok*   No     Ok                         Ok

(ACC)   2M  No

Yamaha:         .   .     .      .      .     .      .      .

6388

NA   Not supported

Ok   Works

No   Does not work

!   Doesn't work in mode 13h

*   See notes below

Notes:

ARK Logic 1000VL/2000PV. Scroll in 256/High/Truecolor modes is in units of

4bytes. So far I haven't figured out how to use the BitBLT engine.

ATI 18800-1 & 28800-6. Horizontal scroll in PK4 mode (and PL4 for 18800-1)

is in units of 2 pixels. The 28800-6 requires extensive reprogramming of

mode 0Dh (timing problems when full memory is enabled).

I suspect that all combinations of the 28800-4/-5/-6 and BIOS id 4 or 5 can

be found. Examples ?

ATI Mach64 VGA. Scroll in PL4 modes is in units of 2 pixels. There are some

HiColor modes (72h,73h,75h), but they are partialy controlled by the Mach64

engine, so have been disabled for now

ATI Mach32 (68800-3). Blit & Line fails 24bit, but works in 4/8/15/16bits.

Mach32 can not tell bytes/scanline or interlace (write only regs)

The VGA mode 13h wraps at 64K.

ATI Mach64 (88800GX). Scroll is in units of 8 bytes. The clock probing works,

but where is the baseline? BitBLT and Line draw does not work in 24bit modes,

works with 32bpp modes. The 16color (packed 4bit) modes shows a blank screen

Avance Logic ALG2101. Blit (Fill) & Line fails in 64Kcolor modes. The cursor

is double height in interlaced modes

Avance Logic ALG2201/2228/2301. Blit, Line & Cursor fails because the graphics

engine registers are memory mapped (but to where ?).

The test for the ALG2228/2301 is suspect. It works for the two cards I have..

Acumos AVGA2 and Cirrus Logic 5402/5420 (rev 1). The cursor does not work in

the 320x200 modes due to the clock doubling

Cirrus Logic CL-GD5426. The cursor does not work in the 320x200 modes due to

the clock doubling, and also not in the 16Mcolor modes.

BitBlt does not work in the 16Mcolor modes (copy ok, fill fails).

BitBLT in 16c modes ?

Cirrus Logic CL-GD5430,CL-GD5434. The cursor does not work in the 320x200

modes due to the clock doubling, and also not in the 16Mcolor modes.

BitBlt does not work in the 16Mcolor modes (copy ok, fill fails), also not in

16color modes and mode 13h.

Note: Not tested in 32bit (RGBa) mode (too little RAM)/missing modes.

Note: On the 5430 the 16color modes blanked (all white)) the 2nd MB.

Cirrus Logic CL-GD6410. Can't id the clocks.

Compaq AVGA:

BitBLT only works in extended 256color modes.

Compaq QVision 1280. The AVGA modes (0-13h,29h,2Eh) wraps at 1Mb. Some modes

(800x600, 640x480 16Mcolors) are broken - BIOS problem. AVGA Blit only works

in mode 2Eh, fails in all other AVGA modes.

Scroll in packed modes (except mode 13h) is in units of 4 bytes

HMC 86c304. Horizontal scroll in 256c modes is in units of 4 bytes.

It is still uncertain if we can distinguish between the '304 and the '314

IIT (VGA). Memory limited to 512K due to scroll & lack of modes. Banking can

access 1Mb, but scroll wraps at 512K.

IIT-14/15/16. Scroll is in units of 4 bytes.

Only tested w/1024x768 8bit modes due to lack of BIOS modes.

Matrox VGA. Mode 13h is redefined to allow full memory access, but the new

mode has a 62 kHz line sync (I.e. we need a /2 on the dot-clock).

Matrix Ultima. The card is ID'd, but memory, DAC and clock is not checked.

No functionality expect for the ID (I.e. no modes...)

MediaVision PG1024. The ID rutine could generate "false positives" and will

not work if the adapter has not been setup for a base I/O address (538h/...)

Also does not check for 768K/2.25M (256color/16Mcolor versions).

All Paradise: Scroll in extended 256color modes is in units of 4 bytes

WD90c31: Cursor is horizontally compressed in 32K/64Kcolor modes. Blit fails

(copy ok, fill fails) in 32K/64K color modes.

WD90c33: Blit & Line fails in 16Mcolor modes

Paradise/WD90c24: Scroll in extended 256color modes is in units of 4 bytes

The cursor moves too slowly vertically in some Flat Panel modes (the 200/350

/400 line ones), probably due to vertical stretching.

Primus P2000. Interlaced modes have a problem scrolling into the 2nd Mb where

the image from the 1st Mb is superimposed on the "real" image from the 2nd

Mb. Blit fails in 32k/64Kcolor modes (copy is ok, fill fails).

Oak-37C,-67. Extended modes and R/W Banks could not be tested due to the low-

spec cards used. There are several different OAK-37 BIOSes with diff. modes!

Oak-087. The hardware cursor only works in 256color modes.

S3 86c924. Cursor has problems in clock doubled modes (mode $D and $13)

BitBLT & Line does not work in 16color (planar) and standard VGA modes

Scroll in 256&Highcolor modes is in units of 4 bytes

S3 86c801. Cursor has problems in clock doubled modes (mode $D and $13)

BitBLT & Line does not work in 16color (planar), standard VGA and 24bit modes

Scroll in 256&Highcolor modes is in units of 4 bytes

S3 86c805p. Cursor has problems in clock doubled modes (mode $D and $13)

and locks up in the packed 16color mode. BitBLT & Line does not work in

16color (planar), standard VGA and 24bit modes

Scroll in 256&Highcolor modes is in units of 4 bytes

S3 86c928. Cursor has problems in clock doubled modes (mode $D and $13)

Scroll in 256/High/Truecolor modes is in units of 4 or 8 bytes

BitBLT & Line does not work in 16color (planar) and standard VGA modes

S3 Vision864. The cursor is broken in all VGA and 16color (planar) modes

Scroll in 256/High/Truecolor modes is in units of 4 or 8 bytes

BitBLT & Line does not work in 16color (planar) and standard VGA modes

S3 Vision964. The cursor is broken in all modes using the S3 cursor (16color

(planar) and some 256color modes), as if the cursor map is stored wrong.

Scroll in 256/High/Truecolor modes is in units of 4 or 8 bytes

BitBLT & Line does not work in 16color (planar) and standard VGA modes

S3 Trio64 (86c764). Cursor has problems in clock doubled modes (mode $D and

$13). Scroll in 256/High/Truecolor modes is in units of 4 or 8 bytes

BitBLT & Line does not work in 16color (planar) and standard VGA modes

Trident. The newer Tridents must have some extra bits for the horizontal

timing registers. This can be seen in the 320x200 24bit mode where the

Horizontal Total rolls over and thus becomes very small.

The 9000i has an onchip 15/16bit DAC, but apparently no one can get it to work

- not even the BIOS/VESA modes. I expect it requires 16bits per pixel clock.

The 9000C, 8900CL and 8900D can not reliably be distinguished from each other.

The 8900B/8900C has a problem with images crossing 512K in 16color modes.

It is ok if the entire image is either below or above 512K

Trident 9440. The clocks are detected from the internal clock chip, but are

off by as much as 10%.

Tseng ET3000AX. Mode 0Dh wraps at 256K. This mode does not appear to be

reprogrammable (unlike mode 13h and the other 16color modes).

Tseng ET4000/W32.

Cursor is too short horizontally in 24bit modes and 15/16bit modes.

Blit: Fill doesn't work in 24bit modes. Copy only works down left!!

Blit doesn't work in 16c modes

Tseng ET4000/W32i revB. Cursor has a coloumn of junk below the actual cursor

in interlaced modes (Happens with the W32i rev B & W32p rev A).

Also cursor is too short horizontally in 15/16/24bit modes and too large

in mode 13h (pixel doubling).

Blit is broken in all modes (Fill appears to work in 8/15/16bit modes, copy

fails in all modes - destination block is often "broken up" or "dirty").

Tseng ET4000/W32p revA. Cursor has a coloumn of junk below the actual cursor

in interlaced modes (Happens with the W32i rev B & W32p rev A).

The BitBlt does not copy "backwards". Lines appears "dotted".

Cursor is too short horizontally in 24bit modes

Tseng ET4000/W32p revB. Blit: Fill doesn't work in 24bit. Blit & lines fails

totally in 16color modes. Lines appears dotted.

Cursor hor. short in 15/16/24 bit modes, too wide in mode 13h.

Some problems w/Analysis due to the Chrontel DAC

Horizontal Scroll in "16bit DAC video path" mode is in units of 2 (or 4?) bytes

Tseng ET4000/W32p revD. Blit: Fill doesn't work in 24bit. Blit & lines fails

totally in 16color modes. Lines appears dotted.

Cursor hor. short in 24 bit modes, too wide in mode 13h.

Horizontal Scroll in "16bit DAC video path" mode is in units of 2 bytes

UMC UM85c408,UM85c418. Some of the modes are weird (BIOS problem).

Video7 HT-208 (actually G2 208). Hardware cursor is horizontally compressed in

256color modes and blank in mode 13h (the last is also true for the HT-216).

Also, I have my doubts as to the clock selection on the '208.

Weitek W5x86,P9100(VGA): Scroll only works in the first 256K (as std VGA).

Horizontal Scroll in ext 256color modes is in units of 4 bytes

DACs tested:

ID   8bit  15b  16b  24b  32b Cursor  MPlex  Clock

Acumos:

ADAC1                                      NA    NA     NA     NA

AT&T:              .     .    .    .    .    .     .      .      .

20c490           Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     NA

20c491           Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     NA

20c492           Ok    NA   Ok   Ok   OK1  NA    NA     NA     NA

20c493                 NA                  NA    NA     NA     NA

20c498                                           NA

21c498                                           NA

22c498                                           NA

20c504

20c505

ATI:               .     .    .    .    .    .     .      .      .

ATI68830                                         NA

ATI68860               Ok   Ok   Ok   Ok   Ok    NA            Ok

ATI68875               pin                       NA

ATI68880

Avance Logic:      .     .    .    .    .    .     .      .      .

ALG1101          Ok*   NA   NA   pin  NA   NA    NA     NA     NA

ALG1201          Ok         Ok   Ok   Ok1        NA     NA     NA

ALG1301

Brooktree:         .     .    .    .    .    .     .      .      .

Bt477            Ok    Ok   NA   NA   NA   NA    NA     NA     NA

Bt481            Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     NA

Bt482                                                   NA

Bt484

Bt485            Ok    Ok   Ok*  Ok*  NA   Ok*   Ok     No     NA

Chrontel:          .     .    .    .    .    .     .      .      .

CH8391           Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     Ok

CH8398           Ok    NA   Ok   Ok   Ok1  NA    NA     No     Ok

Chips&Tech:

64300 Internal   OK         Ok   Ok   Ok1        NA     NA     Ok

Cirrus Logic:      .     .    .    .    .    .     .      .      .

CL-GD5200

542x internal    Ok    NA   Ok   Ok   Ok   NA    NA     NA     Ok

543x internal

Diamond:           .     .    .    .    .    .     .      .      .

SS24             Ok    NA   Ok   Ok   Ok1  NA    NA     NA     NA

IBM:

RGB514

RGB524           Ok

RGB525

RGB526

RGB528

RGB530

RGB561

RGB624

ICS:               .     .    .    .    .    .     .      .      .

5300

5301             Ok    NA   Ok   Ok   OK1  NA    NA     NA     NA

5342

IC Works:

w30c498

w30c516          Ok    Ok   Ok   Ok        Ok    NA     Ok     NA

Paradise/WD:

WD90c24 Intern   Ok         Ok*  Ok*             NA     NA     NA

Oak:               .     .    .    .    .    .     .      .      .

OTI-66

OTI-66HC         Ok    NA   Ok   Ok              NA     NA     NA

MUSIC:             .     .    .    .    .    .     .      .      .

MU9c1710

MU9c1880

MU9c4870

MU9c4910         Ok    NA   Ok   Ok   Ok1  NA    NA     NA     NA

MU9c9910         Ok    NA   Ok   Ok   Ok1  NA    NA     NA     Ok

S3:                .     .    .    .    .    .     .      .      .

86c708 (GenDAC)

86c716 (SDAC)    Ok    NA   Ok   Ok        Ok*   NA            Ok

SGS-Thompson:      .     .    .    .    .    .     .      .      .

STG1700                                          NA

STG1702          Ok    Ok   Ok   Ok   Ok1        NA     NA     NA

STG1703          Ok    Ok                        NA

Sierra:            .     .    .    .    .    .     .      .      .

SC11481                pin       NA   NA   NA    NA     NA

SC11486                NA        NA   NA   NA    NA     NA

SC11488                pin       NA   NA   NA    NA     NA

SC11482                pin       NA   NA   NA    NA     NA

SC11483                NA        NA   NA   NA    NA     NA

SC11484                pin       NA   NA   NA    NA     NA

SC11485                pin            NA   NA    NA     NA

SC11487          Ok    NA   Ok   Ok   NA   NA    NA     NA     NA

SC11489                pin            NA   NA    NA     NA

SC15021          Ok    Ok   Ok   Ok   Ok1  NA           NA

SC15025          Ok    Ok   Ok   Ok   Ok1        NA     NA     NA

TI:                .     .    .    .    .    .     .      .      .

TLC34058

TLC34075         Ok    pin  NA   NA   No   NA    NA     No     NA

TVP3010

TVP3020

TVP3025          Ok    Ok   Ok   Ok   No   No    Ok     No     Ok

TVP3026

Trident:           .     .    .    .    .    .     .      .      .

TKD8001          Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     NA

9000i Internal   Ok    NA   No   No   NA   NA    NA     NA     NA

9200CXr Intern   Ok    Ok   Ok   Ok   Ok1  NA    NA     NA     NA

9400CXi Intern   Ok    NA   Ok   Ok   Ok1  NA    NA     NA     NA

9420DGi Intern   Ok    NA   Ok   Ok   Ok1  NA    NA     NA

9440AGi Intern   Ok                              NA     NA

UMC:               .     .    .    .    .    .     .      .      .

UM70c178         Ok         Ok   Ok   NA   NA    NA     NA     NA

UM70c188         Ok         Ok   Ok   Ok1  NA    NA     NA     NA

Winbond:           .     .    .    .    .    .     .      .      .

W82c490          Ok*   NA   Ok   Ok   Ok1  NA    NA     NA     NA

NA   Not supported

pin  Supported by a pin

Comments:

Ok1  - The DAC is switched into a 24bit mode, but the colors are swapped

around. This may be a problem with the dac mode (BGR rather than RGB)

or with the DAC starting from the wrong byte because the left overscan

area is also seen by the DAC

The Trident TKD8001 can be sent into a "no return" state, which I (and the

BIOS) can't return it from....

The Avance Logic ALG1101 id "ID'd" by assuming that any ALG chips that reports

a standard DAC must have an ALG1101... It is unclear how 16bit mode is set.

The Brooktree Bt484/485 can be switched between 8/15/16/32 bit modes, however

due to the different pixel multiplexing used for these modes the display

timings are often seriously disturbed. Also the DAC must be in "VRAM" mode.

The S3 SDAC can be switched between 8/15/16/32 bit modes, however due to the

different pixel multiplexing used for these modes the display timings are

often seriously disturbed.

Paradise/WD90c24 internal DAC can be switched to 15/16bit mode, but double

clocking mode (16bits/VCLK) must be set for it to work.

The Winbond W82C490 is ID'd as "AT&T 20c492", which is close enough

The ATI 68860 is ID'd from the Mach64 status regs. Switch to 15/16/24/32

bit mode works, but changes the horizontal resolution.

Clock chips:

Id:    Rd:

DCS2824     Ok     NA

ICD20c61    Ok     NA

ICS2595

SC11412

The DCS2824 and ICD20c61 are both identified as "ICD20c61" which is ok as the

DCS2824 is an OEM'd ICS chip. The ICD20c61 and ICD20c61A arte however id'd.

Generel Problems:

Search for modes doesn't understand 2 and 4 color modes.

In general 2 and 4 color modes are only supported partially, and

fixing it is NOT a top priority :-).

Mode 0Fh and 11h (and other 2 color modes) has problems in some cases.

Mode 13h has a pixel double function. Most chips use Master Clock/2

(3C4h index 1 bit 3) for this, but some chips has their own system.

This causes problems with mode analysis.

Some of the extended modes may only be implemented on

some otherwise similar boards, depending on make and BIOS rev.

Utilities:

DMPMEM:

DMPMEM dumps the ROMs to files named DMPxxxx.ROM (where xxxx is

the hex address of the ROM Ie. DMPC000.ROM for VGA).

Interrupt and Font addresses are saved to the file VECTORS.ROM

As Mouse drivers, national keyboard drivers, memory managers

etc can obscure the results the program should be run on a clean

system (empty CONFIG.SYS and AUTOEXEC.BAT say from a floppy).

TABLE:

Takes the register sets dumped to the file REGISTER.VGA and writes

them to REGISTER.TBL. If a register has the same value in all register

sets, the value is only shown the first time.

As the same file names are used every time you will have to delete

and rename the files as needed.

VESALIE:

Can change the version and amount of memory reported by your VESA driver.

Can also hide the VESA driver, so other applications don't see it.

Parameters:  /0 /1 /2  Makes the VESA driver report a specific VESA

VBE version (1.00, 1.01 or 1.02).

/on /off  Turns the VESA VBE on or off. By turning the

VBE off you can prevent applications from using

the VESA interface. Also if you try to load

another VESA driver, it may refuse to install

while the original driver is visible.

/Mxx      Forces the VESA driver to report the specified

amount (in Kbytes) of video memory.

/U        Unload the VESALIE program.

Note that this NOT a VESA driver, it just changes the behavior of

an already installed driver.

SHOWTEST:

Analyses the results in the .tst files generated when WHATVGA is

run with the /a option (WHVGAxxx.TST).

Parameters: SHOWTEST [tstfil] [outfil] [/r]

When run without parameters it lists the test files in the current

directory, showing the filename, chipset, memory, DAC and chipname.

When run with the name of a test file, the test results in this file

are written to the outfile. If no outfile is specified, the name of

the test file is used, with the extension changed to .txt

If the /r switch is used the register dumps for each mode is written

to the outfile (or to .reg).

To do:

- S3 968/IBM RGB524 combo needs a lot of work

- Damn Weitek (W5x86) scroll function

- ET4000/W32 BitBLT/Linedraw

- Attemp W5x86/Trident/ARK BitBLT/Line draw

- Further work on CT64300, AGX-016, P9000, Matrox, PG1024/1280, Appian,

Alliance, SuperMAC and others

- Figure out how to deal with the AGX-16 which has TWO DACs!!